ABC BOAT DATA AND CHECK LIST

MODEL/MANUFACTURER________SANGER DX 2 420 HP_  BOAT NO._____3__(Prescindo)__

SERIAL NO SANDL427H506   BOAT STATUS 	 UNCHANGED  X  NEW or MODIFIED

DIMENSIONS   (SEE RULE 8.02)  OVERALL LENGTH_20’ 4”_ OVERALL WIDTH_88”_

SAFETY CHECK-RUNNING GEAR:

ARE THE SHAFT, COLLAR, STRUT, RUDDER , PROPELLER, NUT AND COTTER KEY SECURELY IN PLACE?   X YES    NO     COMMENTS____________________________

ENGINE	MANUFACTURER/MODEL NO.__8.1L MERCRUISER

CID__496_   HORSEPOWER____420_________________________

PROPELLER	MANUFACTURER___ACME 13.5X15.5_______________________________________

RIGHT HAND____________ LEFT HAND________CUPPED_______

FINS		NUMBER____3____ SIZE__13. 5X15.5_______  Wake plate setting 3/16” +  -_1/16”

PYLON		IS THE PYLON SECURELY INSTALLED AND NON-FLEXING UNDER LOAD?  x  YES     NO

SEATING	DRIVER_____X______   JUDGE___X_____  HEIGHT FRONT__12”_________   REAR___12”_____

SPEEDMETERS 	MANUFACTURER_________________________  NUMBER ________________

STEERING GEAR  	MANUFACTURER ______TELEFLEX__________________

THROTTLE/SHIFT  	MANUFACTURER ______MORSE__________________

GAUGES		YES     NO			SAFETY EQUIPMENT  YES     NO

TACHOMETER           	__X_    ____			FIRE EXTINGUISHER   __X_    ____

FUEL			__X_    ____			BILGE BLOWER	__X -    ____

TEMPERATURE         	__X_    ____		             BILGE PUMP	         	__X_    ____

OIL PRESURE		__X_    ____			C.G. CAPACITY PLATE	__X_    ____

AMPS/VOLTS            	__X_    ____			RUNNING LIGHTS        __X_    ____

FUSES/BREAKERS    	__X_    ____			HORN                            __X -_   ____

HOUR METER            	__X_    ____			MIRROR		__X_    ____

REMOTE TRIM/TILT  	____    _X__

PHOTOGRAPHS				COMMENTS

__X_  Running Gear and Transom Area  	__lower pylon/rope attachment point is 9’5” from the transom.  The tower

_____  Interior and Pylon Area 			attachment point is 4” aft of the lower attachment point

GENERAL COMMENTS  _This is a retest of  a previously approved boat and is only testing a different prop from the one originally tested to give boat owners more choices.